freertos_master_slave

Software Design Document

FreeRTOS Master/Slave State-Machine Simulation

Version:  1.0

Date:  2026-05-13

# Table of Contents

Table of Contents	2

1.  Introduction	3

1.1  Purpose	3

1.2  Scope	3

1.3  Definitions and Abbreviations	3

2.  System Architecture	4

2.1  Architecture Diagram	4

2.2  Component Responsibilities	4

3.  FreeRTOS Configuration	5

3.1  Scheduler and Tick Rate	5

3.2  Task Priorities and Stack Sizes	5

3.3  Key FreeRTOSConfig.h Parameters	6

4.  Device A Design (Master)	6

4.1  Overview	6

4.2  State Machine	6

4.3  Task Implementation	7

4.4  Persistent Fault Detection	7

4.5  Reset Sequence	8

5.  Device B Design (Slave)	8

5.1  Overview	8

5.2  State Machine	8

5.3  Task Implementation	9

5.4  Reset Handling	9

6.  IPC Design	9

6.1  State Queue	10

6.2  Reset Queue	10

6.3  Log Queue	10

6.4  Queue Summary	11

7.  Logger Design	11

7.1  Overview	11

7.2  Log Message Structure	11

7.3  Output Format	11

7.4  Thread Safety	12

8.  Error Handling Strategy	12

9.  Build System	12

9.1  CMake Overview	12

9.2  FreeRTOS Integration	13

9.3  Compiler Flags	13

9.4  Unity Test Framework	13

10.  Running the Simulation	13

10.1  Linux (POSIX port)	13

10.2  Windows (Win32 port)	14

10.3  Running Unit Tests	14

10.4  Configuration Parameters	14

11.  Testing Strategy	14

11.1  Test Structure	14

11.2  Unit Test Philosophy	15

11.3  Integration / Simulation Scenarios	15

12.  Key Design Decisions	15

13.  File and Module Map	16

# 1.  Introduction

This document describes the software design of freertos_master_slave — a FreeRTOS simulation that demonstrates two devices, each implementing a state machine, communicating and synchronising their states via FreeRTOS IPC primitives.  Device A (master) reacts to state changes in Device B (slave); Device B transitions autonomously and does not notify Device A directly.

## 1.1  Purpose

This document is intended for developers who need to understand, extend, or evaluate the freertos_master_slave codebase.  It covers component design, state-machine transition logic, IPC strategy, FreeRTOS configuration, and build/test instructions.

## 1.2  Scope

The design covers four principal software components:

- Device A task  (src/device_a.c) — master state machine and polling logic

- Device B task  (src/device_b.c) — slave state machine and autonomous transitions

- Logger task    (src/logger.c)   — thread-safe log serialiser

- IPC layer      (src/ipc.c)      — FreeRTOS queue handles and message types

Supporting artefacts (FreeRTOSConfig.h, CMakeLists.txt, Unity test suite, simulation entry point) are described where relevant.

## 1.3  Definitions and Abbreviations

| Term | Definition |
| --- | --- |
| Master | Device A; monitors Device B and reacts to its state changes. |
| Slave | Device B; transitions autonomously; does not directly notify Device A. |
| State queue | FreeRTOS queue through which Device B publishes its state after every transition. |
| Reset queue | Single-element FreeRTOS queue through which Device A sends a reset command to Device B. |
| Log queue | FreeRTOS queue through which both device tasks enqueue log messages for the logger task. |
| Persistent FAULT | A FAULT state observed by Device A for >= FAULT_THRESHOLD consecutive polls. |
| Tick | One FreeRTOS scheduler tick; configured to 1 ms (configTICK_RATE_HZ = 1000). |
| POSIX port | FreeRTOS simulator for Linux using POSIX threads (FreeRTOS-Kernel/portable/ThirdParty/GCC/Posix). |
| Win32 port | FreeRTOS simulator for Windows using Win32 threads (FreeRTOS-Kernel/portable/MSVC-MingW). |

# 2.  System Architecture

The simulation runs three concurrent FreeRTOS tasks under the FreeRTOS preemptive scheduler.  All inter-task communication uses FreeRTOS queues; no shared memory with direct concurrent access is used.

## 2.1  Architecture Diagram

+--------------------------------------------------------------------+
|                       FreeRTOS Scheduler                          |
|                                                                    |
|  +----------------------+  state_queue  +----------------------+  |
|  |   Task: Device A     | <------------ |   Task: Device B     |  |
|  |   (master)  pri=3    |               |   (slave)   pri=3    |  |
|  |                      |  reset_queue  |                      |  |
|  |   IDLE               | ------------> |   SLEEP              |  |
|  |   PROCESSING         |               |   ACTIVE             |  |
|  |   ERROR              |               |   FAULT              |  |
|  +----------+-----------+               +-----------+----------+  |
|             |                                       |              |
|             | log_queue (enqueue)                   | log_queue    |
|             +-----------------------+---------------+              |
|                                     |                              |
|                          +----------v-----------+                  |
|                          |   Task: Logger       |                  |
|                          |   pri=1              |                  |
|                          |   stdout output      |                  |
|                          +----------------------+                  |
+--------------------------------------------------------------------+

## 2.2  Component Responsibilities

| Component | File(s) | Responsibility |
| --- | --- | --- |
| Device A task | src/device_a.c/.h | Master state machine. Reads state_queue, evaluates transitions, issues reset via reset_queue, enqueues log messages. |
| Device B task | src/device_b.c/.h | Slave state machine. Transitions autonomously on a timer with pseudo-random logic, posts state to state_queue, handles reset_queue. |
| Logger task | src/logger.c/.h | Dequeues log messages from log_queue and writes them to stdout with tick-count timestamps. |
| IPC layer | src/ipc.c/.h | Declares and initialises all queue handles and message types used for inter-task communication. |
| sim_config.h | src/sim_config.h | Compile-time constants: tick rates, task priorities, stack sizes, fault threshold, simulation duration. |
| FreeRTOSConfig.h | FreeRTOSConfig.h | FreeRTOS kernel configuration: tick rate, heap size, max priorities, stack overflow hook. |
| main.c | src/main.c | Entry point. Creates queues and tasks, starts the scheduler, waits for simulation end, cleans up. |

# 3.  FreeRTOS Configuration

## 3.1  Scheduler and Tick Rate

The scheduler is configured for preemptive operation with time-slicing enabled.  configTICK_RATE_HZ is set to 1000, giving a 1 ms tick period.  All vTaskDelay() and xQueueReceive() timeout values in this document are expressed in ticks, which map directly to milliseconds at this rate.

## 3.2  Task Priorities and Stack Sizes

| Task | Priority | Stack (words) | Rationale |
| --- | --- | --- | --- |
| Device A | 3 | 512 | Higher than logger; equal to Device B so neither starves. |
| Device B | 3 | 512 | Equal to Device A; autonomous transitions must not be blocked. |
| Logger | 1 | 256 | Lower than device tasks; runs in idle time between device task sleeps. |
| Sim timer | 2 | 128 | Soft-timer task that signals simulation end; priority between logger and devices. |

Priority 0 is reserved for the FreeRTOS idle task.  configMAX_PRIORITIES is set to 5 to give headroom for future tasks.

## 3.3  Key FreeRTOSConfig.h Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| configUSE_PREEMPTION | 1 | Preemptive scheduler. |
| configUSE_TIME_SLICING | 1 | Equal-priority tasks share CPU in round-robin. |
| configTICK_RATE_HZ | 1000 | 1 ms tick; convenient for timeout arithmetic. |
| configMAX_PRIORITIES | 5 | Priorities 0–4; idle=0, logger=1, sim_timer=2, devices=3. |
| configMINIMAL_STACK_SIZE | 128 | Words; used by idle task. |
| configTOTAL_HEAP_SIZE | 65536 | 64 KB heap; sufficient for three tasks plus queues. |
| configUSE_MALLOC_FAILED_HOOK | 1 | Asserts on heap exhaustion. |
| configCHECK_FOR_STACK_OVERFLOW | 2 | Method 2: watermark + pattern check on every context switch. |
| configUSE_TRACE_FACILITY | 0 | Disabled; not required for this simulation. |
| INCLUDE_vTaskDelete | 1 | Tasks self-delete on clean exit. |

# 4.  Device A Design (Master)

## 4.1  Overview

Device A runs as a FreeRTOS task that periodically reads from state_queue to learn Device B's current state and drives its own state machine accordingly.  It also monitors for persistent FAULT conditions and issues reset commands when required.

## 4.2  State Machine

Device A implements three states:

| State | Description |
| --- | --- |
| IDLE | Default state. Device A is waiting for Device B to become ACTIVE. |
| PROCESSING | Device B is ACTIVE; Device A is performing its nominal work. |
| ERROR | Device B has been in persistent FAULT; Device A has issued a reset and is waiting for recovery. |

State transition table:

| From state | Condition | To state | Action |
| --- | --- | --- | --- |
| IDLE | B state = ACTIVE | PROCESSING | Log transition. |
| IDLE | Persistent FAULT detected | ERROR | Send reset command to Device B; log event. |
| PROCESSING | B state != ACTIVE and != FAULT | IDLE | Log transition. |
| PROCESSING | Persistent FAULT detected | ERROR | Send reset command to Device B; log event. |
| ERROR | B state = SLEEP (reset acknowledged) | IDLE | Clear fault counter; log recovery. |
| ERROR | Reset timeout elapsed, B still FAULT | ERROR | Log persistent error; re-issue reset. |

## 4.3  Task Implementation

Device A's task function executes the following loop at each iteration (period: DEVICE_A_POLL_MS, default 250 ms):

loop:
  result = xQueueReceive(state_queue, &b_state, pdMS_TO_TICKS(DEVICE_A_POLL_MS))
  if result == pdTRUE:
      process_b_state_update(b_state)   /* update fault counter, drive FSM */
  else:
      /* timeout: no new state from B; re-evaluate with last known state */
      process_b_state_update(last_b_state)
  check_sim_end()

Using xQueueReceive with a timeout rather than a plain vTaskDelay means Device A reacts promptly when Device B posts a state change, while still re-evaluating periodically even when Device B is quiescent.

## 4.4  Persistent Fault Detection

Device A maintains a uint32_t fault_count variable.  On every evaluation cycle:

- If the observed B state is FAULT, fault_count is incremented.

- If the observed B state is not FAULT, fault_count is reset to zero.

- When fault_count reaches FAULT_THRESHOLD (default 3), Device A transitions to ERROR and posts a reset command to reset_queue.

This debouncing prevents a transient FAULT from triggering a reset.  FAULT_THRESHOLD is a compile-time constant in sim_config.h.

## 4.5  Reset Sequence

Device A                           Device B
─────────────────────────────      ──────────────────────────────
fault_count >= FAULT_THRESHOLD
  A state: * -> ERROR
  xQueueSend(reset_queue, RESET)  ──────────────────────────────>
  log "reset issued"
  start reset_timeout timer
                                   xQueueReceive(reset_queue)
                                   B state: FAULT -> SLEEP
                                   xQueueSend(state_queue, SLEEP)
                                   log "reset received"
  xQueueReceive(state_queue)
  b_state = SLEEP
  A state: ERROR -> IDLE
  fault_count = 0
  log "recovery complete"

If reset_queue is full when Device A attempts to send (i.e., a previous reset command has not yet been consumed), xQueueSend returns errQUEUE_FULL and Device A logs the condition without blocking.  The retry occurs on the next evaluation cycle.

# 5.  Device B Design (Slave)

## 5.1  Overview

Device B runs as a FreeRTOS task that autonomously transitions between states on a timer, using a pseudo-random value to choose the next state.  After each transition it posts its new state to state_queue.  It checks reset_queue on every wake-up and returns immediately to SLEEP if a reset command is present.

## 5.2  State Machine

Device B implements three states:

| State | Description |
| --- | --- |
| SLEEP | Device B is inactive. Normal starting state after power-on or reset. |
| ACTIVE | Device B is performing its nominal operation. |
| FAULT | Device B has detected an internal error condition. |

Autonomous state transition table:

| From state | Condition | To state |
| --- | --- | --- |
| SLEEP | rand() % 4 == 0 (25% chance per tick interval) | ACTIVE |
| ACTIVE | rand() % 5 == 0 (20% chance per tick interval) | SLEEP |
| ACTIVE | rand() % 8 == 0 (12.5% chance per tick interval) | FAULT |
| FAULT | rand() % 6 == 0 (16.7% chance per tick interval) | ACTIVE |
| FAULT | Reset command received from Device A | SLEEP |

Probabilities and the state transition interval (DEVICE_B_TICK_MS, default 500 ms) are compile-time constants in sim_config.h.  rand() is seeded with a fixed value (RAND_SEED, default 42) at startup to allow reproducible runs.

## 5.3  Task Implementation

Device B's task function executes the following loop:

loop:
  /* Check for reset command first (non-blocking) */
  if xQueueReceive(reset_queue, &cmd, 0) == pdTRUE:
      transition_to(SLEEP)
      xQueueSend(state_queue, SLEEP, 0)
      log "reset received"
      vTaskDelay(pdMS_TO_TICKS(DEVICE_B_TICK_MS))
      continue

  /* Autonomous transition */
  new_state = compute_next_state(current_state)
  if new_state != current_state:
      transition_to(new_state)
      xQueueSend(state_queue, new_state, 0)   /* non-blocking */
      log transition

  vTaskDelay(pdMS_TO_TICKS(DEVICE_B_TICK_MS))
  check_sim_end()

## 5.4  Reset Handling

The reset_queue is a single-element queue (depth 1) of type reset_cmd_t.  Device B polls it with a zero-tick timeout (non-blocking) at the start of every wake-up cycle.  If a reset command is present, Device B abandons its autonomous transition for that cycle, moves unconditionally to SLEEP, and posts SLEEP to state_queue so that Device A can observe the recovery.

# 6.  IPC Design

All inter-task communication uses FreeRTOS queues.  No direct shared-memory access is used; all state information is passed by value through queue items, ensuring memory safety without requiring explicit mutexes on shared data.

## 6.1  State Queue

Device B writes its state to state_queue after every transition.

| Attribute | Value |
| --- | --- |
| Handle | QueueHandle_t g_state_queue |
| Item type | device_b_state_t (uint8_t enum) |
| Item size | sizeof(device_b_state_t) |
| Depth | 8 (FR-17: minimum 8) |
| Writer | Device B task only |
| Reader | Device A task only |
| Send policy | xQueueSend with timeout 0 (non-blocking); overflow logged but not fatal |
| Receive policy | xQueueReceive with timeout pdMS_TO_TICKS(DEVICE_A_POLL_MS) |

## 6.2  Reset Queue

Device A writes a reset command to reset_queue when it detects a persistent FAULT.

| Attribute | Value |
| --- | --- |
| Handle | QueueHandle_t g_reset_queue |
| Item type | reset_cmd_t (uint8_t, value RESET_CMD_REBOOT) |
| Item size | sizeof(reset_cmd_t) |
| Depth | 1 (at most one outstanding reset at a time) |
| Writer | Device A task only |
| Reader | Device B task only |
| Send policy | xQueueSend with timeout 0 (non-blocking); errQUEUE_FULL logged |
| Receive policy | xQueueReceive with timeout 0 (non-blocking poll every B wake-up) |

## 6.3  Log Queue

Both device tasks enqueue log messages to log_queue; the logger task dequeues and prints them.

| Attribute | Value |
| --- | --- |
| Handle | QueueHandle_t g_log_queue |
| Item type | log_msg_t (struct: tick, source, text[LOG_MSG_MAX_LEN]) |
| Item size | sizeof(log_msg_t) |
| Depth | 32 |
| Writers | Device A task, Device B task |
| Reader | Logger task only |
| Send policy | xQueueSend with timeout 0; message silently dropped on overflow |
| Receive policy | xQueueReceive blocks indefinitely (portMAX_DELAY) |

## 6.4  Queue Summary

| Queue | Type | Depth | Direction | Blocking write? | Blocking read? |
| --- | --- | --- | --- | --- | --- |
| state_queue | device_b_state_t | 8 | B → A | No | Yes (poll timeout) |
| reset_queue | reset_cmd_t | 1 | A → B | No | No (zero timeout) |
| log_queue | log_msg_t | 32 | A,B → Logger | No | Yes (indefinite) |

# 7.  Logger Design

## 7.1  Overview

The logger task runs at priority 1 — lower than both device tasks — and serialises all log output to stdout.  Device tasks never call printf() directly; they enqueue a log_msg_t item and return immediately, keeping their own execution latency independent of I/O speed.

## 7.2  Log Message Structure

typedef struct {
    TickType_t  tick;                      /* xTaskGetTickCount() at enqueue time */
    log_source_t source;                   /* LOG_SRC_DEVICE_A | LOG_SRC_DEVICE_B */
    char         text[LOG_MSG_MAX_LEN];    /* NUL-terminated message string (64 bytes) */
} log_msg_t;

## 7.3  Output Format

Each log line follows the pattern:

[TICK:0000012345] [DEVICE_A] IDLE -> PROCESSING  (B_STATE=ACTIVE)

Example output from a complete fault-recovery cycle:

[TICK:0000000000] [DEVICE_B] SLEEP  (initial state)
[TICK:0000001502] [DEVICE_B] SLEEP -> ACTIVE
[TICK:0000001503] [DEVICE_A] IDLE -> PROCESSING  (B_STATE=ACTIVE)
[TICK:0000003011] [DEVICE_B] ACTIVE -> FAULT
[TICK:0000003012] [DEVICE_A] fault_count=1
[TICK:0000003512] [DEVICE_A] fault_count=2
[TICK:0000004012] [DEVICE_A] fault_count=3  persistent FAULT — reset issued
[TICK:0000004012] [DEVICE_A] PROCESSING -> ERROR
[TICK:0000004514] [DEVICE_B] FAULT -> SLEEP  (reset received)
[TICK:0000004515] [DEVICE_A] ERROR -> IDLE  (recovery complete)

## 7.4  Thread Safety

The log_queue is a FreeRTOS queue and is therefore inherently thread-safe.  The logger task is the sole reader and calls xQueueReceive with portMAX_DELAY, blocking until a message is available.  No mutex is required because printf() is called only from the single logger task.

# 8.  Error Handling Strategy

| Error condition | Detection | Response |
| --- | --- | --- |
| Heap allocation failure (xTaskCreate, xQueueCreate) | configUSE_MALLOC_FAILED_HOOK = 1 | vApplicationMallocFailedHook() asserts; simulation aborts. |
| Stack overflow | configCHECK_FOR_STACK_OVERFLOW = 2 | vApplicationStackOverflowHook() asserts; simulation aborts. |
| state_queue overflow (Device B writes faster than Device A reads) | xQueueSend returns errQUEUE_FULL | Device B logs a warning and discards the oldest state notification. Queue depth 8 makes this extremely unlikely. |
| reset_queue full (Device A issues second reset before Device B processes first) | xQueueSend returns errQUEUE_FULL | Device A logs the condition and retries on the next evaluation cycle. |
| log_queue overflow | xQueueSend returns errQUEUE_FULL | Message silently dropped; simulation continues. Queue depth 32 makes this unlikely under normal operation. |
| Device B does not recover after reset (reset timeout) | Device A checks B state after RESET_TIMEOUT_MS | Device A remains in ERROR state, logs persistent error, and re-issues reset command. |

# 9.  Build System

## 9.1  CMake Overview

A single top-level CMakeLists.txt drives the build.  cmake_minimum_required is 3.18 to use FetchContent_MakeAvailable.  Two targets are produced:

- freertos_sim — the main simulation executable.

- freertos_tests — the Unity unit-test executable (enabled with -DBUILD_TESTS=ON).

## 9.2  FreeRTOS Integration

FreeRTOS is fetched automatically using CMake FetchContent from the official FreeRTOS-Kernel repository (tag v11.1.0) if not found as a system package.  The correct portable layer is selected at configure time:

if(UNIX)
    set(FREERTOS_PORT GCC_POSIX)
    set(FREERTOS_HEAP  heap_3)    # uses system malloc
elseif(WIN32)
    set(FREERTOS_PORT MSVC_MingW)
    set(FREERTOS_HEAP  heap_4)
endif()

FreeRTOSConfig.h is placed in the project root and added to the include path so it is found by the FreeRTOS kernel headers.

## 9.3  Compiler Flags

| Flag | Purpose |
| --- | --- |
| -Wall -Wextra -Wpedantic | Enable all standard warnings (NFR-02). |
| -Wno-unused-parameter | Suppress FreeRTOS portable layer unused-parameter warnings. |
| -std=c11 | Enforce C11 standard (NFR-01). |
| -pthread | Link POSIX thread library on Linux (required by POSIX port). |

## 9.4  Unity Test Framework

Unit tests use the Unity C test framework, fetched via FetchContent.  State-machine logic is compiled into a separate static library (libstatemachine) with no FreeRTOS dependencies, allowing Unity tests to link and exercise it directly.

# 10.  Running the Simulation

## 10.1  Linux (POSIX port)

# Install prerequisites
sudo apt install build-essential cmake git

# Clone and build
cd freertos_master_slave
cmake -B build
cmake --build build

# Run
./build/freertos_sim

## 10.2  Windows (Win32 port)

# Prerequisites: MinGW-w64 or MSVC, CMake 3.18+

cd freertos_master_slave
cmake -B build -G "MinGW Makefiles"
cmake --build build

build\freertos_sim.exe

## 10.3  Running Unit Tests

cmake -B build -DBUILD_TESTS=ON
cmake --build build
./build/freertos_tests

## 10.4  Configuration Parameters

All tunable parameters are compile-time constants in src/sim_config.h.  They can be overridden at cmake configure time:

| Parameter | Default | Description |
| --- | --- | --- |
| SIM_DURATION_MS | 30000 | Total simulation wall-clock time in milliseconds. |
| DEVICE_A_POLL_MS | 250 | Device A evaluation cycle period in milliseconds. |
| DEVICE_B_TICK_MS | 500 | Device B autonomous transition check interval in milliseconds. |
| FAULT_THRESHOLD | 3 | Consecutive FAULT observations before Device A issues a reset. |
| RESET_TIMEOUT_MS | 2000 | Time Device A waits for recovery after issuing reset. |
| RAND_SEED | 42 | Fixed seed for rand(); change for different transition sequences. |
| LOG_MSG_MAX_LEN | 64 | Maximum length of a log message text field (bytes, including NUL). |

# 11.  Testing Strategy

## 11.1  Test Structure

freertos_master_slave/
└── tests/
    ├── test_device_a_sm.c    Unit tests for Device A state machine
    ├── test_device_b_sm.c    Unit tests for Device B state machine
    └── test_ipc_types.c      Tests for message type sizes and enum values

## 11.2  Unit Test Philosophy

State-machine transition functions (device_a_process_event, device_b_next_state) are pure functions that take a current state and an input and return a next state.  They do not call any FreeRTOS API directly, allowing Unity tests to exercise them with no FreeRTOS scheduler running.

Unit tests cover:

- Every valid state transition for Device A and Device B.

- Persistent fault counter increment and reset.

- Fault threshold boundary: fault_count = FAULT_THRESHOLD - 1 (no reset) and FAULT_THRESHOLD (reset).

- Reset command clears Device B state unconditionally regardless of current state.

- Device A stays in ERROR if B does not return to SLEEP within the timeout.

## 11.3  Integration / Simulation Scenarios

The simulation itself serves as an integration test.  The following scenarios are observable in the log output:

| Scenario | Observable in log |
| --- | --- |
| Normal wakeup | B: SLEEP -> ACTIVE; A: IDLE -> PROCESSING |
| Normal sleep | B: ACTIVE -> SLEEP; A: PROCESSING -> IDLE |
| Fault detection | B: ACTIVE -> FAULT; A: fault_count increments to threshold |
| Reset and recovery | A: ERROR; reset issued; B: FAULT -> SLEEP; A: ERROR -> IDLE |
| Reset timeout | A remains in ERROR; re-issues reset after RESET_TIMEOUT_MS |

# 12.  Key Design Decisions

FreeRTOS queues for all IPC — no shared memory.  Queues provide thread-safety, flow control, and natural producer/consumer semantics without requiring explicit mutexes on state variables.

Device B uses non-blocking queue sends.  Device B must not block on I/O so that its autonomous timing remains accurate.  A depth-8 state_queue ensures no messages are dropped under normal conditions.

Device A uses a timed xQueueReceive instead of vTaskDelay + peek.  A timed receive means Device A wakes immediately when Device B posts a new state, giving sub-millisecond reaction time.  The timeout still ensures the evaluation loop runs even when Device B is quiescent.

State-machine logic is FreeRTOS-free (pure functions).  Separating transition logic from IPC allows unit testing without a running scheduler (NFR-06).  The task functions act as thin wrappers: receive input, call the pure transition function, send output.

Persistent fault threshold before reset.  A single FAULT observation could be a transient condition.  Requiring FAULT_THRESHOLD consecutive observations prevents spurious resets and gives Device B a chance to self-recover.

Fixed rand() seed for reproducibility.  A fixed RAND_SEED (default 42) produces the same sequence of Device B transitions on every run, making bugs reproducible and test output predictable.

Dedicated logger task with a log queue.  Printing from multiple tasks concurrently would interleave output.  A single logger task serialises stdout writes.  Device tasks pay only the cost of a non-blocking queue send, keeping their timing tight.

FetchContent for FreeRTOS and Unity.  Avoids requiring the developer to manually clone submodules while still pinning to a specific tagged release for reproducibility.

# 13.  File and Module Map

freertos_master_slave/
├── CMakeLists.txt           Top-level build; FetchContent for FreeRTOS and Unity
├── FreeRTOSConfig.h         Kernel configuration (tick rate, heap, priorities)
├── README.md                Setup, build, and run instructions
├── REQUIREMENTS.docx        Requirements specification (this project)
├── DESIGN.docx              This document
├── src/
│   ├── main.c               Entry point: queue/task creation, scheduler start, cleanup
│   ├── device_a.c           Device A FreeRTOS task and state-machine logic
│   ├── device_a.h           Device A public API and state enum
│   ├── device_b.c           Device B FreeRTOS task and autonomous transition logic
│   ├── device_b.h           Device B public API and state enum
│   ├── ipc.c                Queue handle definitions and initialisation
│   ├── ipc.h                Queue handle declarations and message type definitions
│   ├── logger.c             Logger task: dequeues log_msg_t and prints to stdout
│   ├── logger.h             log_msg_t type, log_source_t enum, logger_log() helper
│   └── sim_config.h         Compile-time tuning constants
└── tests/
    ├── test_device_a_sm.c   Unity tests: Device A state-machine transitions
    ├── test_device_b_sm.c   Unity tests: Device B state-machine transitions
    └── test_ipc_types.c     Unity tests: message type sizes and enum values

# Potential Improvements and Extensions

The following extensions could be made to the simulation to increase its realism, portability, or observability. Each is independent and can be implemented without restructuring the existing codebase.

•  Additional devices — The queue-based IPC architecture scales naturally to more devices. Adding a Device C requires only a new task, a new queue pair, and entries in ipc.h — no changes to existing device code are needed.

•  Real hardware target — The application is portable to bare-metal FreeRTOS targets (e.g. STM32 Cortex-M). Swap the POSIX/Win32 simulator port for the Cortex-M port, replace heap_3 (which wraps system malloc) with heap_4 for deterministic allocation, and update FreeRTOSConfig.h for the target clock and interrupt controller. Application code and state-machine logic require no changes.

•  Persistent logging — The logger task currently writes to stdout. Redirecting its output to a file descriptor or a ring buffer would allow post-run analysis and replay without modifying the device tasks.

•  Runtime fault injection — A signal handler (SIGUSR1) or a dedicated CLI command could force Device B into FAULT on demand. This would enable targeted testing of the fault-detection and recovery paths without waiting on the pseudo-random number generator.

•  Software watchdog — A watchdog task could monitor that each device task produces at least one queue message within a configurable timeout. A missed heartbeat would indicate task starvation or stack corruption and could trigger a controlled shutdown.