freertos_master_slave

Requirements Specification

FreeRTOS Master/Slave State-Machine Simulation

Version:  1.0

Date:  2026-05-13

# Table of Contents

Table of Contents	2

1  Purpose and Scope	3

2  System Overview	3

2.1  Architecture Diagram	3

3  Functional Requirements	4

3.1  FreeRTOS Environment	4

3.2  Device A State Machine (Master)	4

3.3  Device B State Machine (Slave)	4

3.4  Inter-device Communication	5

3.5  Fault Handling and Recovery	5

3.6  Logging	5

3.7  Simulation Control	6

4  Non-functional Requirements	6

5  Constraints and Assumptions	6

6  Glossary	7

# 1  Purpose and Scope

This document specifies the functional and non-functional requirements for the freertos_master_slave simulation project. The simulation demonstrates two FreeRTOS tasks — Device A (master) and Device B (slave) — each implementing a state machine and communicating via FreeRTOS IPC primitives. The project targets both Linux and Windows hosted FreeRTOS environments.

# 2  System Overview

The system consists of two simulated devices running as concurrent FreeRTOS tasks:

- Device A (master): Polls or receives event notifications about Device B's state and reacts accordingly.

- Device B (slave): Autonomously transitions between its own states based on a timer or pseudo-random condition and does not directly notify Device A.

A FreeRTOS queue carries state-change events from Device B to Device A. A dedicated logger task records all state transitions and error events.

## 2.1  Architecture Diagram

+-------------------------------------------------------------+
|                    FreeRTOS Scheduler                       |
|                                                             |
|  +------------------+   State Queue   +------------------+ |
|  |  Task: Device A  | <-------------- |  Task: Device B  | |
|  |  (master)        |                 |  (slave)         | |
|  |                  |   Reset Queue   |                  | |
|  |  IDLE            | --------------> |  SLEEP           | |
|  |  PROCESSING      |                 |  ACTIVE          | |
|  |  ERROR           |                 |  FAULT           | |
|  +------------------+                 +------------------+ |
|          |                                     |            |
|          +------------------+------------------+            |
|                             |                               |
|                  +----------v---------+                     |
|                  |   Task: Logger     |                     |
|                  +--------------------+                     |
+-------------------------------------------------------------+

Device B publishes state-change events to a FreeRTOS queue (state queue). Device A reads that queue to track Device B's current state. Device A sends reset commands to Device B via a separate single-element queue (reset queue). Both tasks enqueue log messages to a logger task that serialises output to stdout.

# 3  Functional Requirements

## 3.1  FreeRTOS Environment

| ID | Requirement |
| --- | --- |
| FR-01 | The simulation shall run on FreeRTOS using the POSIX (Linux) or Windows simulator port. |
| FR-02 | Each device shall execute as a separate FreeRTOS task with its own stack and priority. |
| FR-03 | A dedicated logger task shall run at lower priority than both device tasks. |

## 3.2  Device A State Machine (Master)

| ID | Requirement |
| --- | --- |
| FR-04 | Device A shall implement three states: IDLE, PROCESSING, and ERROR. |
| FR-05 | Device A shall start in the IDLE state on initialisation. |
| FR-06 | Device A shall transition from IDLE to PROCESSING when it detects that Device B is in the ACTIVE state. |
| FR-07 | Device A shall transition to the ERROR state when it detects that Device B is in the FAULT state. |
| FR-08 | Device A shall transition from ERROR back to IDLE after successfully resetting Device B. |
| FR-09 | Device A shall transition from PROCESSING back to IDLE when Device B leaves the ACTIVE state. |

## 3.3  Device B State Machine (Slave)

| ID | Requirement |
| --- | --- |
| FR-10 | Device B shall implement three states: SLEEP, ACTIVE, and FAULT. |
| FR-11 | Device B shall start in the SLEEP state on initialisation. |
| FR-12 | Device B shall transition autonomously between states based on a configurable timer interval and/or pseudo-random condition. |
| FR-13 | Device B shall not directly notify Device A of state changes; state is communicated via the state queue. |
| FR-14 | Device B shall accept a reset command from Device A and return to the SLEEP state when received. |

## 3.4  Inter-device Communication

| ID | Requirement |
| --- | --- |
| FR-15 | Device B shall write its current state to a FreeRTOS queue after every state transition. |
| FR-16 | Device A shall read from the state queue to learn Device B's current state. |
| FR-17 | The state queue depth shall be sufficient to avoid dropping notifications under normal operation (minimum depth: 8). |
| FR-18 | The reset command from Device A to Device B shall be sent via a separate single-element FreeRTOS queue. |

## 3.5  Fault Handling and Recovery

| ID | Requirement |
| --- | --- |
| FR-19 | Device A shall detect a persistent FAULT condition if the FAULT state is observed for a configurable number of consecutive polls (default: 3). |
| FR-20 | On detecting a persistent FAULT, Device A shall issue a reset command to Device B and log the event. |
| FR-21 | If Device B does not recover within a configurable timeout after a reset, Device A shall remain in ERROR and continue logging. |

## 3.6  Logging

| ID | Requirement |
| --- | --- |
| FR-22 | Every state transition for both Device A and Device B shall be logged with a timestamp (FreeRTOS tick count). |
| FR-23 | Every reset command issued by Device A shall be logged. |
| FR-24 | Log output shall be written to stdout in a human-readable, timestamped format. |
| FR-25 | The logger shall be thread-safe; device tasks shall enqueue log messages rather than writing to stdout directly. |

## 3.7  Simulation Control

| ID | Requirement |
| --- | --- |
| FR-26 | The simulation shall run for a configurable duration (default: 30 seconds of wall-clock time) before exiting cleanly. |
| FR-27 | All tasks shall be deleted and all FreeRTOS resources freed before the process exits. |

# 4  Non-functional Requirements

| ID | Requirement |
| --- | --- |
| NFR-01 | The implementation shall be written in C11. |
| NFR-02 | The code shall compile without warnings under gcc -Wall -Wextra -Wpedantic. |
| NFR-03 | The simulation shall build and run on Ubuntu 22.04+ and Windows 10+ without modification. |
| NFR-04 | All FreeRTOS objects (tasks, queues) shall be created at startup; no dynamic creation shall occur during steady-state operation. |
| NFR-05 | Stack usage for each task shall be sized conservatively; a stack overflow hook shall be implemented. |
| NFR-06 | State-machine logic shall be structured to allow unit testing independently of FreeRTOS. |

# 5  Constraints and Assumptions

| ID | Requirement |
| --- | --- |
| CA-01 | FreeRTOS source shall be obtained from the official FreeRTOS kernel repository (v10.x or later). |
| CA-02 | No OS-specific APIs other than FreeRTOS and the POSIX/Windows simulator port shall be used in state-machine logic. |
| CA-03 | Pseudo-random state transitions in Device B shall use rand() seeded with a fixed value to allow reproducible test runs. |
| CA-04 | The project is a simulation; no real hardware is required. |

# 6  Glossary

| Term | Definition |
| --- | --- |
| Master | Device A; the controlling device that monitors and reacts to Device B. |
| Slave | Device B; the monitored device that transitions autonomously. |
| POSIX port | The FreeRTOS simulator port for Linux using POSIX threads. |
| Windows port | The FreeRTOS simulator port for Windows using Win32 threads. |
| Tick | The fundamental FreeRTOS time unit; one scheduler tick interval. |
| State queue | FreeRTOS queue used by Device B to publish state-change events to Device A. |
| Reset queue | Single-element FreeRTOS queue used by Device A to send a reset command to Device B. |
| Persistent FAULT | A FAULT state in Device B that has been observed for >= the configured consecutive detection threshold. |